# ATTN:

IN RESPONSE TO YOUR REQUEST FOR SPECS ON PART, 4/0 GA BATTERY CABLE, PLEASE FIND THE FOLLOWING:

- GA:  4/0 GA

- STRANDING:  19X22/.0224 BC

- CLASS: H

- COPPER DIA: .525

- OVERALL O.D:  .740 O.D.

- TOLERANCE: +/-.015

- JACKET TYPE:  105 DEGREE PVC

- COPPER CONTENT:  664#/M’

- STANDARD PUT-UP:  500’ PER 30” REEL

- CIR MILS: 209,735

THIS IS TO CONFIRM THAT PART 4/0 GA SGT LISTED MEETS SAE J1127.

______________________________

ANDY HARWOOD

VICE PRESIDENT